EE 5193 — FPGA and HDL

Summer 2026

Electrical and Computer Engineering Department

University of Texas at San Antonio

A Multi-Cycle 16-Bit RISC Processor

on the Nexys A7

Course Project Report

Sergio Arreola

July 10, 2026

Table of Contents

(Right-click and choose Update Field to build the table of contents.)

# 2. Problem Statement

The goal of this project is to design, simulate, and implement a small Reduced Instruction Set Computer (RISC) on the Digilent Nexys A7 board (Artix-7 XC7A100T-1CSG324C) using Vivado. The processor executes 16-bit instructions whose leftmost four bits are the opcode, and every functional block in the datapath/controller diagram has to be written as its own Verilog module.

The controller is specified to have exactly two inputs — the 16-bit IR value and a flag RF_Rp_zero from the register file indicating that the value on read port P is zero — and it must sequence every instruction through the same four steps:

1.  Fetch — read the instruction at memory location PC into the IR,

2.  Decode — interpret the opcode and register/address fields,

3.  Execute — perform the operation,

4.  Update PC — point the PC at the next memory location.

Memory locations 201 and 202 hold initial operand values. The demonstration program below loads them, computes with them, and stores three results at locations 203, 204, and 205, which must appear on the board's 7-segment display once the program has finished:

| # | Mnemonic | Encoding |
| --- | --- | --- |
| 0 | LW r5, 201 | 1001_0101_1100_1001 |
| 1 | LW r6, 202 | 1001_0110_1100_1010 |
| 2 | ADD r7, r5, r6 | 0000_0111_0101_0110 |
| 3 | SW r7, 203 | 1010_0111_1100_1011 |
| 4 | LI r8, 250 | 1000_1000_1111_1010 |
| 5 | SUB r4, r8, r5 | 0001_0100_1000_0101 |
| 6 | SW r4, 204 | 1010_0100_1100_1100 |
| 7 | SRA r3, r7 | 0110_0011_0111_0000 * |
| 8 | XOR r2, r3, r4 | 0100_0010_0011_0100 |
| 9 | SW r2, 205 | 1010_0010_1100_1101 |

* The handout gives 0110_0010_0111_0000 for this instruction, whose destination field decodes to r2, not r3. That contradicts both the mnemonic and the following XOR, which reads r3. I use the corrected encoding here; Section 10.1 describes how I found the discrepancy in simulation.

The deliverables are the Verilog modules, a block diagram, ASM-D charts, a controller output table per opcode, simulation results, and this report.

# 3. Approach

## 3.1 Multi-cycle organization

I implemented the processor as a multi-cycle machine: one controller FSM state per architectural step, so every instruction takes exactly four states (Fetch, Decode, Execute, UpdatePC). This maps one-to-one onto the four-step process the assignment requires, which makes the controller easy to specify, easy to draw as an ASM-D chart, and easy to check in a waveform — the state register tells you exactly which step you are looking at. A single-cycle design would have needed separate instruction and data memories (the assignment wants one memory holding both program and data), and a pipelined design is unjustifiable for a ten-instruction demo.

The FSM has a fifth, terminal state, Halt. The demo program is ten instructions long and the assignment says results are displayed “once all instructions have executed,” so the machine needs a defined way to stop. I assigned opcode 1111 to a HALT instruction, placed one at address 10 (right after the program), and made the controller park in the Halt state with a done output high. done gates the display and drives LED0, and the testbench uses it to know when to check results.

## 3.2 One unified memory

Program and data share a single 256 × 16-bit memory. Addresses 0–10 hold the program (plus HALT), 201/202 hold the input operands, and 203–205 receive the results. A unified memory means one address bus and a single two-way address mux: during Fetch the address is the PC, and during the Execute step of LW/SW it is the 8-bit address field IR[7:0]. Since the machine is multi-cycle, instruction fetch and data access never happen in the same state, so one port is enough and there is no structural hazard.

The CPU port of the memory uses a combinational read with a synchronous write. This is a deliberate choice: with an asynchronous read, the fetched word is available at the memory output in the same state the address is applied, so Fetch and the LW Execute step each fit in a single FSM state and the four-step model stays exact. Vivado infers distributed (LUT) RAM for this style, which costs a few hundred LUTs at 256 × 16 — cheap on an XC7A100T. My first version used a registered read that maps to block RAM, and the one-cycle read latency quietly broke the four-step model; Section 10.2 covers that bug and why I settled on distributed RAM.

Initial contents (the ten instructions, the HALT, and the operands 25 and 35 at 201/202) are loaded from program.mem with $readmemb, so the same image feeds both simulation and synthesis.

## 3.3 Controller

The controller matches the assignment's interface exactly: its only inputs (besides clock, reset, and the clock enable) are IR[15:0] and RF_Rp_zero. It is a five-state FSM (the four steps plus Halt). All outputs are generated in a single combinational block with default assignments first, so no latches are inferred; the opcode IR[15:12] selects the Execute-step outputs, and RF_Rp_zero is consulted in exactly one place.

About that flag: none of the ten demo instructions is conditional, but the assignment specifies RF_Rp_zero as a controller input, so I gave it a real job rather than leaving a dangling port. I defined a conditional branch, JZ rs, addr (opcode 1100, same field layout as SW): if the register on read port P is zero, the PC loads IR[7:0] in the UpdatePC step instead of incrementing. The demo program never executes a JZ, but the path is implemented, simulated, and documented in the controller table, and it is the reason the PC has a load input as well as an increment.

## 3.4 Reuse of previously developed modules

Two blocks in this design are adaptations of modules I built earlier in the course rather than new work:

ALU/shifter (alu.v): the arithmetic unit from the earlier combinational-design lab, trimmed to the four operations this ISA needs (ADD, SUB, XOR, and a one-bit arithmetic right shift). The shift is the sign-replicating {a[15], a[15:1]} slice from the lab shifter; I removed the ops this project doesn't use rather than carrying dead logic through synthesis.

7-segment scan driver (sevenseg_driver.v): the multiplexed 8-digit driver from the display lab. For this project I added a per-digit blanking mask (to insert dark separator digits between the three result bytes) and a “dash mode” that shows -- -- -- while the CPU is still running, so the display state distinguishes “running” from “done” at a glance.

Everything else — PC, IR, register file, memory, controller, and the top level — was written new for this project, one module per box in the block diagram.

## 3.5 Register file and instruction formats

The register file is 16 × 16-bit with two combinational read ports (P and Q) and one synchronous write port. Port P also produces the RF_Rp_zero flag (rp_data == 0) required by the controller. The instruction formats decode as:

| Format | Opcode(s) | Bits [15:12] | Bits [11:8] | Bits [7:0] |
| --- | --- | --- | --- | --- |
| R-type | ADD 0000, SUB 0001, XOR 0100 | opcode | rd | rs [7:4], rt [3:0] |
| Shift | SRA 0110 | opcode | rd | rs [7:4], unused [3:0] |
| LI | 1000 | opcode | rd | imm8 (zero-extended) |
| LW | 1001 | opcode | rd | addr8 |
| SW | 1010 | opcode | rs | addr8 |
| JZ | 1100 | opcode | rs | addr8 |
| HALT | 1111 | opcode | unused | unused |

One decode wrinkle: for SW and JZ the source register sits in bits [11:8], where every other format keeps the destination. Rather than add a third read port, read port P's address comes through a small mux (rf_rp_addr_sel): normally IR[7:4] (rs), but IR[11:8] during the Execute step of SW/JZ. The store data path is then simply port P into the memory's write-data input.

LI zero-extends its 8-bit immediate; with values up to 250 in the demo program, sign extension would corrupt anything above 127. SRA shifts by one bit with sign replication, which matches the expected result (SRA of 60 → 30).

## 3.6 Initial data and hand-calculated results

I chose mem[201] = 25 and mem[202] = 35. Every intermediate value is distinct and each instruction visibly changes state, so a single waveform pass verifies all of them:

| # | Instruction | Effect | Value |
| --- | --- | --- | --- |
| 0 | LW r5, 201 | r5 ← mem[201] | 25 |
| 1 | LW r6, 202 | r6 ← mem[202] | 35 |
| 2 | ADD r7, r5, r6 | r7 ← 25 + 35 | 60 |
| 3 | SW r7, 203 | mem[203] ← r7 | 60 (0x3C) |
| 4 | LI r8, 250 | r8 ← 250 | 250 |
| 5 | SUB r4, r8, r5 | r4 ← 250 − 25 | 225 |
| 6 | SW r4, 204 | mem[204] ← r4 | 225 (0xE1) |
| 7 | SRA r3, r7 | r3 ← 60 >>> 1 | 30 |
| 8 | XOR r2, r3, r4 | r2 ← 0x001E ^ 0x00E1 | 255 |
| 9 | SW r2, 205 | mem[205] ← r2 | 255 (0xFF) |

The XOR is worth spelling out: 30 = 0001_1110, 225 = 1110_0001; the operands are bitwise complements over the low byte, so the result is 1111_1111 = 255. The display therefore ends at 3C E1 FF.

## 3.7 Clocking, reset, and the display path

The whole design runs in a single 100 MHz clock domain. The CPU is throttled by a clock enable — a one-cycle pulse every 100 clocks (1 MHz architectural rate) — rather than a divided clock. Every sequential element in the core (PC, IR, register file write, memory write, FSM state) qualifies its update with en, while the display logic runs at the full 100 MHz. My first attempt used a divided clock and turned into a two-clock-domain design with CDC warnings; Section 10.5 explains the switch. At 1 MHz the program finishes in about 44 µs — instant to the eye, but the rate is trivially adjustable if I ever want to single-step with a slower enable.

Reset is the center pushbutton (BTNC), taken through a two-flop synchronizer. Because it is a level reset and the FSM only advances once every 100 clocks, contact bounce just stretches the reset interval; no separate debouncer is needed.

For output, the memory has a second, read-only combinational port that the CPU never uses. A small scanner in the top level continuously cycles it through addresses 203/204/205 and latches the three words into result registers. The display shows the low byte of each in hex across the eight digits as 3C  E1  FF (digit pairs 7–6, 4–3, 1–0, with digits 5 and 2 blanked as separators). Until done goes high the enabled digits show dashes, so the board visibly distinguishes reset/running from finished. I chose hex over decimal because three 3-digit decimal numbers need nine digits — one more than the board has — while hex is unambiguous, and the report and testbench give the decimal equivalents.

# 4. Block Diagram

Every box below is one Verilog module (one file each); risc_core.v instantiates the six core blocks plus the three datapath muxes, and risc_top.v adds the board-level blocks. (I redraw this figure by hand for the submitted diagram; the ASCII version is the authoritative wiring reference.)

~~~~~~~~~~~ risc_core ~~~~~~~~~~~

IR[15:0]

+------------------------------------------------------------+

|                        CONTROLLER                          |<-- RF_Rp_zero

|        FSM: Fetch -> Decode -> Execute -> UpdatePC         |--> done

+------------------------------------------------------------+

| pc_ld   | ir_ld   | mem_addr_sel | rf_we        | alu_op

| pc_inc  |         | mem_we       | rf_wdata_sel |

v         v         v              | rf_rp_addr_sel

v

+------+  pc_out[7:0]   +---+

|  PC  |--------------->| M |  mem_addr[7:0]  +----------------+

+------+           .--->| U |---------------->|     MEMORY     |

^               |    | X |                 |  256 x 16 RAM  |

| IR[7:0]       |    +---+                 |  (port A: CPU) |

| (JZ target)   |   sel = mem_addr_sel     |                |

|               |                          |  din     dout  |

|   IR[7:0] ----+                          +----------------+

|   (addr8)                                    ^        |

|                                     Rp_data  |        | mem_dout[15:0]

|                                    (SW data) |        |

|                                              |        v

|    +----------------+   ir_out[15:0]         |   (to IR and

+----|       IR       |<-----------------------+    write mux)

+----------------+

| | | |

rd=IR[11:8]  | | | |  rs=IR[7:4], rt=IR[3:0], imm8/addr8

v v v v

+---+   +------------------------+

IR[11:8]   |     REGISTER FILE      |

-->| M |-->| 16 x 16, 2R / 1W       |

IR[7:4]|   |  rp_addr    rq_addr    |

-->| U |   |  waddr <- IR[11:8]     |--> RF_Rp_zero

| X |   |  wdata <- write mux    |    (to controller)

+---+   |  rp_data     rq_data   |

sel =    +------------------------+

rf_rp_         |            |

addr_sel       v            v

+------------------+

|       ALU        |

| ADD SUB XOR SRA  |

+------------------+

| alu_y

v

+-------------+  <---- mem_dout (LW)

|  WRITE MUX  |  <---- {8'd0, IR[7:0]} (LI imm)

+-------------+

|  rf_wdata (back up to register file)

v

sel = rf_wdata_sel: 00 ALU / 01 MEM / 10 IMM

~~~~~~~~~~~ risc_top (board level) ~~~~~~~~~~~

clk 100MHz --> [CLOCK-ENABLE DIVIDER /100] --> en (to every core block)

BTNC ------> [2-FF SYNC] --> reset

MEMORY port B <--> [RESULT SCANNER: addr 203/204/205, 3 latches]

|

v

[SEVENSEG_DRIVER  8-digit scan]---> AN[7:0], CA..CG, DP

done ---------------------------------------------> LED0, dash-mode select

# 5. ASM-D Charts

## 5.1 Overall four-step loop

Drawing conventions: rectangles are states (Moore outputs listed beside them), angle-bracket diamonds are decisions, and rounded (parenthesized) boxes are conditional outputs valid within the state they hang off. Every register transfer written next to a state happens on the clock-enable edge that leaves that state.

reset

|

v

+-------------+

+--->|    FETCH    |   mem_addr_sel=0 (addr = PC), ir_ld=1

|    +-------------+   IR <- mem[PC]

|           |

|           v

|    +-------------+

|    |   DECODE    |   (no outputs asserted; RF read ports

|    +-------------+    and RF_Rp_zero settle off the new IR)

|           |

|           v

|    +-------------+

|    |   EXECUTE   |   per-opcode outputs

|    +-------------+   (Section 5.2 and table in Section 6)

|           |

|           v

|   < opcode == 1111 (HALT)? >---yes---> +----------+

|           | no                         |   HALT   |--+

|           v                            |  done=1  |<-+

|    +-------------+                     +----------+

|    |  UPDATE PC  |

|    +-------------+

|           |

|           v

|   < opcode == JZ  AND  RF_Rp_zero? >

|           | no                | yes

|           v                   v

|    ( pc_inc = 1 )      ( pc_ld = 1, rf_rp_addr_sel = 1 )

|      PC <- PC + 1        PC <- IR[7:0]

|           |                   |

+-----------+-------------------+

In words: out of reset the FSM sits in FETCH with the address mux pointed at the PC and ir_ld asserted, so the instruction word is captured into the IR on the state-exit edge. DECODE asserts nothing; it exists so the register file's combinational read ports (and the RF_Rp_zero flag) settle a full state before EXECUTE uses them, and it is where the opcode fields become valid to the controller. EXECUTE raises the per-opcode signals from the Section 6 table. The HALT test hangs off EXECUTE: opcode 1111 sends the machine to the terminal HALT state, which holds done high forever (only reset leaves it). UPDATE PC is the one place RF_Rp_zero matters — a JZ whose source register read zero loads the branch target IR[7:0] into the PC; every other case increments.

## 5.2 Execute-step branching per opcode

EXECUTE

|

< opcode = IR[15:12] >

|

+--- 0000 ADD:  alu_op=00, rf_wdata_sel=ALU, rf_we=1    rd <- Rp + Rq

+--- 0001 SUB:  alu_op=01, rf_wdata_sel=ALU, rf_we=1    rd <- Rp - Rq

+--- 0100 XOR:  alu_op=10, rf_wdata_sel=ALU, rf_we=1    rd <- Rp ^ Rq

+--- 0110 SRA:  alu_op=11, rf_wdata_sel=ALU, rf_we=1    rd <- {Rp[15],Rp[15:1]}

+--- 1000 LI :  rf_wdata_sel=IMM, rf_we=1               rd <- {8'd0, IR[7:0]}

+--- 1001 LW :  mem_addr_sel=1, rf_wdata_sel=MEM,       rd <- mem[IR[7:0]]

|               rf_we=1

+--- 1010 SW :  mem_addr_sel=1, rf_rp_addr_sel=1,       mem[IR[7:0]] <- Rp

|               mem_we=1                                (Rp = r[IR[11:8]])

+--- 1100 JZ :  rf_rp_addr_sel=1                        (branch resolves in

|                                                        UPDATE PC)

+--- 1111 HALT: (no outputs; next state = HALT)

# 6. Controller Output Table

Signals: PC_ld, PC_inc (program counter), IR_ld (instruction register), MemSel = mem_addr_sel (0 = PC, 1 = IR[7:0]), Mem_we, RF_we, WSel = rf_wdata_sel (00 = ALU, 01 = memory, 10 = immediate), RpSel = rf_rp_addr_sel (0 = IR[7:4], 1 = IR[11:8]), ALU = alu_op (00 ADD, 01 SUB, 10 XOR, 11 SRA), done. Unlisted values are 0; “–” means don't-care (the default 0/00 is emitted so the logic stays latch-free).

Steps common to every opcode:

| Step | PC_ld | PC_inc | IR_ld | MemSel | Mem_we | RF_we | WSel | RpSel | ALU | done |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fetch | 0 | 0 | 1 | 0 | 0 | 0 | – | – | – | 0 |
| Decode | 0 | 0 | 0 | – | 0 | 0 | – | – | – | 0 |
| UpdatePC (all but taken JZ) | 0 | 1 | 0 | – | 0 | 0 | – | – | – | 0 |
| UpdatePC (JZ, RF_Rp_zero=1) | 1 | 0 | 0 | – | 0 | 0 | – | 1 | – | 0 |
| Halt (terminal state) | 0 | 0 | 0 | – | 0 | 0 | – | – | – | 1 |

Execute step, by opcode:

| Opcode | Instr | PC_ld | PC_inc | IR_ld | MemSel | Mem_we | RF_we | WSel | RpSel | ALU | done |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | ADD | 0 | 0 | 0 | – | 0 | 1 | 00 | 0 | 00 | 0 |
| 0001 | SUB | 0 | 0 | 0 | – | 0 | 1 | 00 | 0 | 01 | 0 |
| 0100 | XOR | 0 | 0 | 0 | – | 0 | 1 | 00 | 0 | 10 | 0 |
| 0110 | SRA | 0 | 0 | 0 | – | 0 | 1 | 00 | 0 | 11 | 0 |
| 1000 | LI | 0 | 0 | 0 | – | 0 | 1 | 10 | – | – | 0 |
| 1001 | LW | 0 | 0 | 0 | 1 | 0 | 1 | 01 | – | – | 0 |
| 1010 | SW | 0 | 0 | 0 | 1 | 1 | 0 | – | 1 | – | 0 |
| 1100 | JZ | 0 | 0 | 0 | – | 0 | 0 | – | 1 | – | 0 |
| 1111 | HALT | 0 | 0 | 0 | – | 0 | 0 | – | – | – | 0 |

Notes: SW and JZ set RpSel = 1 because their source register is in IR[11:8]. JZ asserts nothing else in Execute — it only steers read port P so that RF_Rp_zero reflects the tested register; the actual PC action happens in UpdatePC. HALT's Execute step is a no-op whose only effect is the transition into the terminal Halt state.

# 7. Verilog Code

All modules are Verilog-2001, one file per block, written to synthesize cleanly in Vivado (no latches, single clock domain, memory styles chosen to infer distributed RAM). The testbench is simulation-only.

## 7.1 controller.v — the FSM controller

//-----------------------------------------------------------------------------

// controller.v -- four-state FSM controller (Fetch / Decode / Execute / UpdatePC)

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Inputs, exactly as the assignment specifies: the 16-bit IR value and the

// RF_Rp_zero flag from the register file. The FSM walks the four required

// steps for every instruction and branches on the opcode (IR[15:12]) only

// inside the Execute and UpdatePC steps. RF_Rp_zero is consulted only by

// the conditional branch JZ; a HALT opcode parks the machine in a fifth,

// terminal state and raises `done` for the testbench and the display.

//

// State register updates are qualified by `en` (the CPU clock enable), so

// each architectural step lasts one enable period. Outputs are a pure

// function of (state, opcode, rp_zero) -- Moore-style states with a Mealy

// branch decision in UpdatePC -- generated in one combinational block with

// default assignments first, so no latches are inferred.

//-----------------------------------------------------------------------------

module controller (

input  wire        clk,

input  wire        reset,          // synchronous, active high

input  wire        en,             // CPU clock enable

input  wire [15:0] ir,             // instruction register value

input  wire        rf_rp_zero,     // register file read-port-P == 0 flag

// PC controls

output reg         pc_ld,          // load branch target (taken JZ)

output reg         pc_inc,         // PC <- PC + 1

// IR control

output reg         ir_ld,          // capture memory output into IR

// Memory controls

output reg         mem_addr_sel,   // 0: addr = PC (fetch), 1: addr = IR[7:0]

output reg         mem_we,         // memory write (SW)

// Register file controls

output reg         rf_we,          // register write (ADD/SUB/XOR/SRA/LI/LW)

output reg  [1:0]  rf_wdata_sel,   // 00: ALU, 01: memory, 10: zero-ext imm8

output reg         rf_rp_addr_sel, // 0: Rp addr = IR[7:4], 1: IR[11:8] (SW/JZ)

// ALU control

output reg  [1:0]  alu_op,         // 00 ADD, 01 SUB, 10 XOR, 11 SRA

// Status

output reg         done            // program finished (HALT reached)

);

// Opcodes (IR[15:12])

localparam [3:0] OP_ADD  = 4'b0000,

OP_SUB  = 4'b0001,

OP_XOR  = 4'b0100,

OP_SRA  = 4'b0110,

OP_LI   = 4'b1000,

OP_LW   = 4'b1001,

OP_SW   = 4'b1010,

OP_JZ   = 4'b1100,   // branch if Rp == 0 (uses RF_Rp_zero)

OP_HALT = 4'b1111;

// Write-data mux encodings

localparam [1:0] WD_ALU = 2'b00,

WD_MEM = 2'b01,

WD_IMM = 2'b10;

// States: the four required steps plus a terminal halt state

localparam [2:0] S_FETCH  = 3'd0,

S_DECODE = 3'd1,

S_EXEC   = 3'd2,

S_UPDATE = 3'd3,

S_HALT   = 3'd4;

reg [2:0] state, next_state;

wire [3:0] opcode = ir[15:12];

// ---- state register -----------------------------------------------

always @(posedge clk) begin

if (reset)

state <= S_FETCH;

else if (en)

state <= next_state;

end

// ---- next-state logic ----------------------------------------------

always @* begin

case (state)

S_FETCH:  next_state = S_DECODE;

S_DECODE: next_state = S_EXEC;

S_EXEC:   next_state = (opcode == OP_HALT) ? S_HALT : S_UPDATE;

S_UPDATE: next_state = S_FETCH;

S_HALT:   next_state = S_HALT;

default:  next_state = S_FETCH;

endcase

end

// ---- output logic ----------------------------------------------------

// Defaults first so every path is covered and nothing infers a latch.

always @* begin

pc_ld          = 1'b0;

pc_inc         = 1'b0;

ir_ld          = 1'b0;

mem_addr_sel   = 1'b0;      // default: address = PC

mem_we         = 1'b0;

rf_we          = 1'b0;

rf_wdata_sel   = WD_ALU;

rf_rp_addr_sel = 1'b0;      // default: Rp reads rs = IR[7:4]

alu_op         = 2'b00;

done           = 1'b0;

case (state)

// Step 1: instruction at mem[PC] into IR

S_FETCH: begin

mem_addr_sel = 1'b0;

ir_ld        = 1'b1;

end

// Step 2: opcode/fields settle; register file read ports and

// the RF_Rp_zero flag become valid. No register is written.

S_DECODE: begin

// no asserted outputs -- decode is combinational off the IR

end

// Step 3: per-opcode work

S_EXEC: begin

case (opcode)

OP_ADD: begin

alu_op       = 2'b00;

rf_wdata_sel = WD_ALU;

rf_we        = 1'b1;

end

OP_SUB: begin

alu_op       = 2'b01;

rf_wdata_sel = WD_ALU;

rf_we        = 1'b1;

end

OP_XOR: begin

alu_op       = 2'b10;

rf_wdata_sel = WD_ALU;

rf_we        = 1'b1;

end

OP_SRA: begin

alu_op       = 2'b11;

rf_wdata_sel = WD_ALU;

rf_we        = 1'b1;

end

OP_LI: begin

rf_wdata_sel = WD_IMM;

rf_we        = 1'b1;

end

OP_LW: begin

mem_addr_sel = 1'b1;     // address = IR[7:0]

rf_wdata_sel = WD_MEM;

rf_we        = 1'b1;

end

OP_SW: begin

mem_addr_sel   = 1'b1;   // address = IR[7:0]

rf_rp_addr_sel = 1'b1;   // Rp reads rs = IR[11:8]

mem_we         = 1'b1;

end

OP_JZ: begin

rf_rp_addr_sel = 1'b1;   // Rp reads rs = IR[11:8];

// branch resolves in UpdatePC

end

default: ;                   // OP_HALT and unused opcodes

endcase

end

// Step 4: next sequential address, or the branch target for a

// taken JZ (this is the one place RF_Rp_zero is used)

S_UPDATE: begin

if (opcode == OP_JZ && rf_rp_zero) begin

rf_rp_addr_sel = 1'b1;       // keep rs selected so the

pc_ld          = 1'b1;       // flag stays valid

end else begin

pc_inc = 1'b1;

end

end

S_HALT: begin

done = 1'b1;

end

default: ;

endcase

end

endmodule

## 7.2 pc.v — program counter

//-----------------------------------------------------------------------------

// pc.v -- 8-bit program counter

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Holds the address of the current instruction. Supports synchronous load

// (for taken branches) and increment (UpdatePC step). Load has priority over

// increment; the controller never asserts both. All updates are qualified by

// the CPU clock enable `en` so the whole core can be throttled from the

// 100 MHz board clock without a derived clock.

//-----------------------------------------------------------------------------

module pc (

input  wire       clk,

input  wire       reset,   // synchronous, active high

input  wire       en,      // CPU clock enable

input  wire       ld,      // load pc_in (branch target)

input  wire       inc,     // pc <= pc + 1

input  wire [7:0] pc_in,   // branch target from IR[7:0]

output reg  [7:0] pc_out

);

always @(posedge clk) begin

if (reset)

pc_out <= 8'd0;

else if (en) begin

if (ld)

pc_out <= pc_in;

else if (inc)

pc_out <= pc_out + 8'd1;

end

end

endmodule

## 7.3 ir.v — instruction register

//-----------------------------------------------------------------------------

// ir.v -- 16-bit instruction register

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Captures the memory output word at the end of the Fetch step. The

// controller decodes ir_out directly; the fields are

//   ir_out[15:12] opcode

//   ir_out[11:8]  rd (or rs for SW/JZ)

//   ir_out[7:4]   rs

//   ir_out[3:0]   rt

//   ir_out[7:0]   imm8 / addr8

//-----------------------------------------------------------------------------

module ir (

input  wire        clk,

input  wire        reset,   // synchronous, active high

input  wire        en,      // CPU clock enable

input  wire        ld,      // capture ir_in

input  wire [15:0] ir_in,   // memory read data

output reg  [15:0] ir_out

);

always @(posedge clk) begin

if (reset)

ir_out <= 16'd0;

else if (en && ld)

ir_out <= ir_in;

end

endmodule

## 7.4 regfile.v — register file with RF_Rp_zero flag

//-----------------------------------------------------------------------------

// regfile.v -- 16 x 16-bit register file, 2 read ports / 1 write port

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Read ports are combinational (the Decode step gives the read data a full

// state period to settle before the Execute step uses it). The write port is

// synchronous and qualified by the CPU clock enable. Writes use a

// non-blocking assignment so a read and a write in the same cycle return the

// OLD register value -- see the report, "Problems encountered", for the

// blocking-assignment bug this replaced.

//

// rp_zero is the flag required by the assignment: it is high whenever the

// value currently presented on read port P is zero. The controller samples

// it during Execute to resolve the conditional branch (JZ).

//-----------------------------------------------------------------------------

module regfile (

input  wire        clk,

input  wire        en,        // CPU clock enable

input  wire        we,        // write enable (Execute step of writing ops)

input  wire [3:0]  waddr,     // destination register rd

input  wire [15:0] wdata,     // from write-data mux (ALU / mem / imm)

input  wire [3:0]  rp_addr,   // read port P address

input  wire [3:0]  rq_addr,   // read port Q address

output wire [15:0] rp_data,

output wire [15:0] rq_data,

output wire        rp_zero    // flag to controller: rp_data == 0

);

reg [15:0] rf [0:15];

integer i;

initial begin

for (i = 0; i < 16; i = i + 1)

rf[i] = 16'd0;

end

always @(posedge clk) begin

if (en && we)

rf[waddr] <= wdata;

end

assign rp_data = rf[rp_addr];

assign rq_data = rf[rq_addr];

assign rp_zero = (rp_data == 16'd0);

endmodule

## 7.5 alu.v — ALU (reused from earlier lab)

//-----------------------------------------------------------------------------

// alu.v -- 16-bit ALU: ADD, SUB, XOR, SRA

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Adapted from the ALU/shifter developed in the earlier combinational-logic

// lab; trimmed to the four operations this instruction set needs. Purely

// combinational. SRA shifts operand A right by one with sign replication

// (operand B is ignored for that op).

//-----------------------------------------------------------------------------

module alu (

input  wire [15:0] a,       // register file read port P

input  wire [15:0] b,       // register file read port Q

input  wire [1:0]  op,      // 00 ADD, 01 SUB, 10 XOR, 11 SRA

output reg  [15:0] y

);

localparam [1:0] ALU_ADD = 2'b00,

ALU_SUB = 2'b01,

ALU_XOR = 2'b10,

ALU_SRA = 2'b11;

always @* begin

case (op)

ALU_ADD: y = a + b;

ALU_SUB: y = a - b;

ALU_XOR: y = a ^ b;

ALU_SRA: y = {a[15], a[15:1]};   // arithmetic shift right by 1

default: y = 16'd0;

endcase

end

endmodule

## 7.6 memory.v — unified 256 x 16 memory

//-----------------------------------------------------------------------------

// memory.v -- unified 256 x 16-bit instruction/data memory

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// One memory holds both the program (addresses 0..10) and the data

// (201/202 in, 203..205 out), so a single address mux switches the CPU

// between fetching (addr = PC) and load/store (addr = IR[7:0]).

//

// Port A is the CPU port: combinational read, synchronous write. The

// combinational read is deliberate -- it lets Fetch and LW each complete in

// one FSM state. Vivado infers distributed (LUT) RAM for this, which is

// cheap at 256x16. I originally wrote a registered-read version that maps

// to block RAM, but the one-cycle read latency broke the four-state execute

// model (see "Problems encountered" in the report).

//

// Port B is a read-only combinational port used only by the display logic

// in the top level to scan mem[203..205]; the CPU never touches it.

//

// Initial contents come from program.mem via $readmemb: the ten demo

// instructions plus a HALT at 0..10, and the operands 25/35 at 201/202.

//-----------------------------------------------------------------------------

module memory #(

parameter MEM_INIT_FILE = "program.mem"

) (

input  wire        clk,

input  wire        en,       // CPU clock enable

input  wire        we,       // write enable (Execute step of SW)

input  wire [7:0]  addr,     // port A address (PC or IR[7:0])

input  wire [15:0] din,      // store data (register file port P)

output wire [15:0] dout,     // port A read data

input  wire [7:0]  addr_b,   // port B address (display scan)

output wire [15:0] dout_b    // port B read data

);

reg [15:0] ram [0:255];

integer i;

initial begin

for (i = 0; i < 256; i = i + 1)

ram[i] = 16'd0;

$readmemb(MEM_INIT_FILE, ram);

end

always @(posedge clk) begin

if (en && we)

ram[addr] <= din;

end

assign dout   = ram[addr];

assign dout_b = ram[addr_b];

endmodule

## 7.7 program.mem — memory image (program + data)

// program.mem -- initial memory image for the EE 5193 RISC project

// Loaded with $readmemb. Addresses are hex (@ directives), data is binary.

//

// Program (addresses 0..9), HALT at 10:

@00

1001010111001001   // 00: LW  r5, 201      r5 <- mem[201] = 25

1001011011001010   // 01: LW  r6, 202      r6 <- mem[202] = 35

0000011101010110   // 02: ADD r7, r5, r6   r7 <- 25 + 35  = 60

1010011111001011   // 03: SW  r7, 203      mem[203] <- 60

1000100011111010   // 04: LI  r8, 250      r8 <- 250

0001010010000101   // 05: SUB r4, r8, r5   r4 <- 250 - 25 = 225

1010010011001100   // 06: SW  r4, 204      mem[204] <- 225

0110001101110000   // 07: SRA r3, r7       r3 <- 60 >>> 1 = 30

//     NOTE: handout listed 0110_0010_0111_0000 (rd = r2),

//     which contradicts the mnemonic and the following

//     XOR that reads r3. Corrected rd field to 0011.

0100001000110100   // 08: XOR r2, r3, r4   r2 <- 30 ^ 225 = 255

1010001011001101   // 09: SW  r2, 205      mem[205] <- 255

1111000000000000   // 10: HALT

//

// Data (addresses 201 = 0xC9, 202 = 0xCA):

@C9

0000000000011001   // 201: 25

0000000000100011   // 202: 35

## 7.8 risc_core.v — datapath + controller wiring

//-----------------------------------------------------------------------------

// risc_core.v -- datapath + controller wired together (board-independent)

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Every box in the block diagram is its own module instance; this file is

// only wiring plus the three datapath muxes (memory address, register file

// write data, register file Rp read address). Keeping the core free of any

// board logic lets the same module sit under both the testbench (en tied

// high) and risc_top (en driven by the clock-enable divider).

//-----------------------------------------------------------------------------

module risc_core #(

parameter MEM_INIT_FILE = "program.mem"

) (

input  wire        clk,

input  wire        reset,     // synchronous, active high

input  wire        en,        // CPU clock enable

output wire        done,      // HALT reached

// display-side memory read port (used by risc_top, idle in simulation)

input  wire [7:0]  disp_addr,

output wire [15:0] disp_data

);

// ---- interconnect ----------------------------------------------------

wire [7:0]  pc_out;

wire [15:0] ir_out;

wire [15:0] mem_dout;

wire [15:0] rf_rp_data, rf_rq_data;

wire [15:0] alu_y;

wire        rf_rp_zero;

// control signals

wire        pc_ld, pc_inc, ir_ld;

wire        mem_addr_sel, mem_we;

wire        rf_we, rf_rp_addr_sel;

wire [1:0]  rf_wdata_sel;

wire [1:0]  alu_op;

// ---- datapath muxes ---------------------------------------------------

// memory address: PC during Fetch, IR[7:0] during LW/SW Execute

wire [7:0] mem_addr = mem_addr_sel ? ir_out[7:0] : pc_out;

// register file read port P address: rs field, except SW/JZ where the

// source register lives in bits [11:8]

wire [3:0] rp_addr = rf_rp_addr_sel ? ir_out[11:8] : ir_out[7:4];

// register file write data: ALU result / memory word / zero-extended imm8

reg [15:0] rf_wdata;

always @* begin

case (rf_wdata_sel)

2'b01:   rf_wdata = mem_dout;                 // LW

2'b10:   rf_wdata = {8'd0, ir_out[7:0]};      // LI

default: rf_wdata = alu_y;                    // ADD/SUB/XOR/SRA

endcase

end

// ---- module instances --------------------------------------------------

controller u_ctrl (

.clk            (clk),

.reset          (reset),

.en             (en),

.ir             (ir_out),

.rf_rp_zero     (rf_rp_zero),

.pc_ld          (pc_ld),

.pc_inc         (pc_inc),

.ir_ld          (ir_ld),

.mem_addr_sel   (mem_addr_sel),

.mem_we         (mem_we),

.rf_we          (rf_we),

.rf_wdata_sel   (rf_wdata_sel),

.rf_rp_addr_sel (rf_rp_addr_sel),

.alu_op         (alu_op),

.done           (done)

);

pc u_pc (

.clk    (clk),

.reset  (reset),

.en     (en),

.ld     (pc_ld),

.inc    (pc_inc),

.pc_in  (ir_out[7:0]),     // JZ branch target

.pc_out (pc_out)

);

ir u_ir (

.clk    (clk),

.reset  (reset),

.en     (en),

.ld     (ir_ld),

.ir_in  (mem_dout),

.ir_out (ir_out)

);

regfile u_rf (

.clk     (clk),

.en      (en),

.we      (rf_we),

.waddr   (ir_out[11:8]),   // rd

.wdata   (rf_wdata),

.rp_addr (rp_addr),

.rq_addr (ir_out[3:0]),    // rt

.rp_data (rf_rp_data),

.rq_data (rf_rq_data),

.rp_zero (rf_rp_zero)

);

alu u_alu (

.a  (rf_rp_data),

.b  (rf_rq_data),

.op (alu_op),

.y  (alu_y)

);

memory #(.MEM_INIT_FILE(MEM_INIT_FILE)) u_mem (

.clk    (clk),

.en     (en),

.we     (mem_we),

.addr   (mem_addr),

.din    (rf_rp_data),      // SW store data

.dout   (mem_dout),

.addr_b (disp_addr),

.dout_b (disp_data)

);

endmodule

## 7.9 sevenseg_driver.v — 7-segment scan driver (reused from earlier lab)

//-----------------------------------------------------------------------------

// sevenseg_driver.v -- 8-digit multiplexed 7-segment scan driver (Nexys A7)

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Reused from my earlier display lab, with two additions for this project:

// a per-digit blanking mask (digit_en) and a dash mode used while the CPU

// is still running. Anodes and cathodes are both active low on the Nexys A7.

//

// Scan rate: the digit select comes from bits [19:17] of a free-running

// counter at 100 MHz, so each digit is lit for 2^17 * 10 ns = 1.31 ms and

// the full 8-digit frame refreshes at ~95 Hz -- fast enough not to flicker,

// slow enough that the anode drivers settle (see "Problems encountered"

// for the ghosting I got with a faster scan and no blanking).

//

// seg bit order: seg[6:0] = {CG,CF,CE,CD,CC,CB,CA}, i.e. seg[0] = CA.

//-----------------------------------------------------------------------------

module sevenseg_driver (

input  wire        clk,        // 100 MHz board clock

input  wire        reset,      // synchronous, active high

input  wire [31:0] digits,     // 8 hex nibbles, digit 7 = digits[31:28]

input  wire [7:0]  digit_en,   // 1 = display this digit, 0 = blank

input  wire        show_dash,  // 1 = enabled digits show '-'

output reg  [7:0]  an,         // anodes, active low

output reg  [6:0]  seg,        // cathodes CG..CA, active low

output wire        dp          // decimal point, active low (kept off)

);

assign dp = 1'b1;              // decimal point never used

// ---- scan counter -----------------------------------------------------

reg [19:0] scan_cnt;

always @(posedge clk) begin

if (reset)

scan_cnt <= 20'd0;

else

scan_cnt <= scan_cnt + 20'd1;

end

wire [2:0] digit_sel = scan_cnt[19:17];

// ---- current nibble ---------------------------------------------------

reg [3:0] nibble;

always @* begin

case (digit_sel)

3'd0: nibble = digits[3:0];

3'd1: nibble = digits[7:4];

3'd2: nibble = digits[11:8];

3'd3: nibble = digits[15:12];

3'd4: nibble = digits[19:16];

3'd5: nibble = digits[23:20];

3'd6: nibble = digits[27:24];

3'd7: nibble = digits[31:28];

endcase

end

// ---- hex-to-segment decode (active low, {g,f,e,d,c,b,a}) --------------

reg [6:0] hex_seg;

always @* begin

case (nibble)

4'h0: hex_seg = 7'b1000000;

4'h1: hex_seg = 7'b1111001;

4'h2: hex_seg = 7'b0100100;

4'h3: hex_seg = 7'b0110000;

4'h4: hex_seg = 7'b0011001;

4'h5: hex_seg = 7'b0010010;

4'h6: hex_seg = 7'b0000010;

4'h7: hex_seg = 7'b1111000;

4'h8: hex_seg = 7'b0000000;

4'h9: hex_seg = 7'b0010000;

4'hA: hex_seg = 7'b0001000;

4'hB: hex_seg = 7'b0000011;

4'hC: hex_seg = 7'b1000110;

4'hD: hex_seg = 7'b0100001;

4'hE: hex_seg = 7'b0000110;

4'hF: hex_seg = 7'b0001110;

endcase

end

localparam [6:0] SEG_DASH  = 7'b0111111;   // segment G only

localparam [6:0] SEG_BLANK = 7'b1111111;

// ---- registered outputs (anode and cathode switch together) -----------

always @(posedge clk) begin

if (reset) begin

an  <= 8'hFF;

seg <= SEG_BLANK;

end else begin

if (digit_en[digit_sel]) begin

an  <= ~(8'd1 << digit_sel);

seg <= show_dash ? SEG_DASH : hex_seg;

end else begin

an  <= 8'hFF;                  // blank digit: no anode driven

seg <= SEG_BLANK;

end

end

end

endmodule

## 7.10 risc_top.v — board top level

//-----------------------------------------------------------------------------

// risc_top.v -- board top level for the Nexys A7-100T

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Wires the CPU core to the board: 100 MHz clock in, BTNC as reset,

// mem[203]/mem[204]/mem[205] out on the 7-segment display in hex as

//      [203] _ [204] _ [205]   ->   "3C E1 FF"

// (digits 7-6, 4-3, 1-0; digits 5 and 2 blanked as separators). While the

// program is still running the display shows dashes; LED0 lights when the

// HALT instruction is reached.

//

// Clocking: everything runs on the raw 100 MHz clock. The CPU is throttled

// with a clock ENABLE that pulses once per 100 cycles (1 MHz architectural

// rate) instead of a divided clock -- my first attempt used a divided clock

// as a second clock domain and produced CDC warnings and a failing timing

// setup; the clock-enable version keeps one clean clock domain (see

// "Problems encountered").

//-----------------------------------------------------------------------------

module risc_top (

input  wire       clk,     // 100 MHz board clock (E3)

input  wire       btnC,    // center pushbutton = reset, active high

output wire [7:0] an,      // 7-seg anodes, active low

output wire [6:0] seg,     // 7-seg cathodes CG..CA, active low

output wire       dp,      // decimal point, active low

output wire       led0     // done indicator

);

// ---- reset synchronizer ------------------------------------------------

// BTNC is asynchronous; two flops synchronize it into the clock domain.

// The FSM only samples state changes every 100 clocks (cpu_en), so any

// residual contact bounce (~ms) just holds reset a little longer --

// no separate debouncer is needed for a level-sensitive reset.

reg [1:0] rst_sync;

always @(posedge clk)

rst_sync <= {rst_sync[0], btnC};

wire reset = rst_sync[1];

// ---- CPU clock enable: 1 pulse every 100 clocks (1 MHz) ----------------

reg [6:0] ce_cnt;

reg       cpu_en;

always @(posedge clk) begin

if (reset) begin

ce_cnt <= 7'd0;

cpu_en <= 1'b0;

end else if (ce_cnt == 7'd99) begin

ce_cnt <= 7'd0;

cpu_en <= 1'b1;

end else begin

ce_cnt <= ce_cnt + 7'd1;

cpu_en <= 1'b0;

end

end

// ---- CPU core -----------------------------------------------------------

wire        done;

wire [7:0]  disp_addr;

wire [15:0] disp_data;

risc_core #(.MEM_INIT_FILE("program.mem")) u_core (

.clk       (clk),

.reset     (reset),

.en        (cpu_en),

.done      (done),

.disp_addr (disp_addr),

.disp_data (disp_data)

);

assign led0 = done;

// ---- result capture: scan mem[203..205] through memory port B -----------

reg  [1:0]  scan_idx;

reg  [15:0] result [0:2];      // result[0]=mem[203], [1]=204, [2]=205

assign disp_addr = 8'd203 + {6'd0, scan_idx};

always @(posedge clk) begin

if (reset) begin

scan_idx  <= 2'd0;

result[0] <= 16'd0;

result[1] <= 16'd0;

result[2] <= 16'd0;

end else begin

result[scan_idx] <= disp_data;

scan_idx         <= (scan_idx == 2'd2) ? 2'd0 : scan_idx + 2'd1;

end

end

// ---- display formatting --------------------------------------------------

// digit:   7    6    5    4    3    2    1    0

// shows: 203h 203l  --  204h 204l  --  205h 205l

wire [31:0] digits = { result[0][7:0], 4'h0,

result[1][7:0], 4'h0,

result[2][7:0] };

sevenseg_driver u_disp (

.clk       (clk),

.reset     (reset),

.digits    (digits),

.digit_en  (8'b1101_1011),   // digits 5 and 2 are blank separators

.show_dash (~done),          // dashes until the program halts

.an        (an),

.seg       (seg),

.dp        (dp)

);

endmodule

## 7.11 risc_tb.v — self-checking testbench (simulation only)

//-----------------------------------------------------------------------------

// risc_tb.v -- self-checking testbench for the EE 5193 RISC core

// EE 5193 FPGA and HDL, Summer 2026 -- RISC project

// Sergio Arreola

//

// Runs the ten-instruction demo program on risc_core with the clock enable

// tied high (one FSM state per clock), waits for `done`, then checks

// mem[203..205] and the involved registers against the hand-calculated

// values:

//     mem[201]=25, mem[202]=35

//     r5=25, r6=35, r7=60  -> mem[203] = 60  (0x3C)

//     r8=250, r4=225       -> mem[204] = 225 (0xE1)

//     r3=30,  r2=255       -> mem[205] = 255 (0xFF)

// Prints PASS/FAIL per check and a summary. A watchdog kills the run if

// `done` never rises (e.g. a broken FSM loop).

//-----------------------------------------------------------------------------

`timescale 1ns / 1ps

module risc_tb;

reg  clk = 1'b0;

reg  reset;

wire done;

integer errors = 0;

// Device under test: core only, display port parked at 0

risc_core #(.MEM_INIT_FILE("program.mem")) dut (

.clk       (clk),

.reset     (reset),

.en        (1'b1),

.done      (done),

.disp_addr (8'd0),

.disp_data ()               // unused in simulation

);

// 100 MHz clock

always #5 clk = ~clk;

// ---- check helpers ------------------------------------------------------

task check_mem;

input [7:0]  addr;

input [15:0] expect;

begin

if (dut.u_mem.ram[addr] === expect)

$display("PASS  mem[%0d] = %0d (0x%02h)",

addr, dut.u_mem.ram[addr], dut.u_mem.ram[addr]);

else begin

$display("FAIL  mem[%0d] = %0d, expected %0d",

addr, dut.u_mem.ram[addr], expect);

errors = errors + 1;

end

end

endtask

task check_reg;

input [3:0]  rnum;

input [15:0] expect;

begin

if (dut.u_rf.rf[rnum] === expect)

$display("PASS  r%0d = %0d", rnum, dut.u_rf.rf[rnum]);

else begin

$display("FAIL  r%0d = %0d, expected %0d",

rnum, dut.u_rf.rf[rnum], expect);

errors = errors + 1;

end

end

endtask

// ---- stimulus ------------------------------------------------------------

initial begin

$display("=== EE 5193 RISC testbench ===");

reset = 1'b1;

repeat (4) @(posedge clk);

reset = 1'b0;

wait (done);

repeat (2) @(posedge clk);

$display("--- program halted at PC = %0d after %0t ---",

dut.u_pc.pc_out, $time);

check_reg(4'd5, 16'd25);      // LW  r5, 201

check_reg(4'd6, 16'd35);      // LW  r6, 202

check_reg(4'd7, 16'd60);      // ADD r7, r5, r6

check_reg(4'd8, 16'd250);     // LI  r8, 250

check_reg(4'd4, 16'd225);     // SUB r4, r8, r5

check_reg(4'd3, 16'd30);      // SRA r3, r7

check_reg(4'd2, 16'd255);     // XOR r2, r3, r4

check_mem(8'd203, 16'd60);    // SW r7, 203

check_mem(8'd204, 16'd225);   // SW r4, 204

check_mem(8'd205, 16'd255);   // SW r2, 205

if (errors == 0)

$display("=== ALL CHECKS PASSED ===");

else

$display("=== %0d CHECK(S) FAILED ===", errors);

$finish;

end

// ---- watchdog -------------------------------------------------------------

initial begin

#10000;   // 10 us >> 11 instructions x 4 states x 10 ns

$display("FAIL  watchdog timeout: done never asserted");

$finish;

end

endmodule

## 7.12 nexys_a7_100t.xdc — Nexys A7-100T constraints

# nexys_a7_100t.xdc -- pin constraints for the EE 5193 RISC project

# Sergio Arreola, EE 5193 FPGA and HDL, Summer 2026

# Pin names/sites taken from the Digilent Nexys A7-100T master XDC.

## 100 MHz system clock

set_property -dict { PACKAGE_PIN E3  IOSTANDARD LVCMOS33 } [get_ports clk]

create_clock -period 10.000 -name sys_clk -waveform {0 5} [get_ports clk]

## Center pushbutton = reset

set_property -dict { PACKAGE_PIN N17 IOSTANDARD LVCMOS33 } [get_ports btnC]

## LED0 = done indicator

set_property -dict { PACKAGE_PIN H17 IOSTANDARD LVCMOS33 } [get_ports led0]

## 7-segment cathodes (active low), seg[0]=CA ... seg[6]=CG

set_property -dict { PACKAGE_PIN T10 IOSTANDARD LVCMOS33 } [get_ports {seg[0]}]

set_property -dict { PACKAGE_PIN R10 IOSTANDARD LVCMOS33 } [get_ports {seg[1]}]

set_property -dict { PACKAGE_PIN K16 IOSTANDARD LVCMOS33 } [get_ports {seg[2]}]

set_property -dict { PACKAGE_PIN K13 IOSTANDARD LVCMOS33 } [get_ports {seg[3]}]

set_property -dict { PACKAGE_PIN P15 IOSTANDARD LVCMOS33 } [get_ports {seg[4]}]

set_property -dict { PACKAGE_PIN T11 IOSTANDARD LVCMOS33 } [get_ports {seg[5]}]

set_property -dict { PACKAGE_PIN L18 IOSTANDARD LVCMOS33 } [get_ports {seg[6]}]

## Decimal point (active low)

set_property -dict { PACKAGE_PIN H15 IOSTANDARD LVCMOS33 } [get_ports dp]

## 7-segment anodes (active low)

set_property -dict { PACKAGE_PIN J17 IOSTANDARD LVCMOS33 } [get_ports {an[0]}]

set_property -dict { PACKAGE_PIN J18 IOSTANDARD LVCMOS33 } [get_ports {an[1]}]

set_property -dict { PACKAGE_PIN T9  IOSTANDARD LVCMOS33 } [get_ports {an[2]}]

set_property -dict { PACKAGE_PIN J14 IOSTANDARD LVCMOS33 } [get_ports {an[3]}]

set_property -dict { PACKAGE_PIN P14 IOSTANDARD LVCMOS33 } [get_ports {an[4]}]

set_property -dict { PACKAGE_PIN T14 IOSTANDARD LVCMOS33 } [get_ports {an[5]}]

set_property -dict { PACKAGE_PIN K2  IOSTANDARD LVCMOS33 } [get_ports {an[6]}]

set_property -dict { PACKAGE_PIN U13 IOSTANDARD LVCMOS33 } [get_ports {an[7]}]

## Configuration

set_property CFGBVS VCCO [current_design]

set_property CONFIG_VOLTAGE 3.3 [current_design]

# 8. Simulation Results and Waveform Discussion

I ran the self-checking testbench (risc_tb.v) as a Vivado behavioral simulation with the clock enable tied high, so each FSM state lasts one 10 ns clock. The whole program — eleven fetch-decode-execute-update sequences counting the HALT — completes in under 500 ns. The testbench waits for done, then compares every result register and the three output memory locations against the hand calculation from Section 3.6. The subsections below walk through the waveforms for three representative instructions and the testbench console output; each figure frame marks where the corresponding Vivado screen capture goes.

## 8.1 A load, end to end: LW r5, 201 (instruction 0)

[ PLACE SCREENSHOT HERE ] Vivado waveform, LW r5,201 fetch-to-writeback. Signals: clk, state, pc_out, mem_addr, mem_dout, ir_out, rf_wdata, rf_we, u_rf.rf[5].

Figure 8-1: Vivado waveform, LW r5,201 fetch-to-writeback. Signals: clk, state, pc_out, mem_addr, mem_dout, ir_out, rf_wdata, rf_we, u_rf.rf[5].

This is the first instruction out of reset, and the waveform shows all four steps in order:

Fetch (state = 0): mem_addr_sel = 0, so mem_addr equals pc_out = 0. The memory read is combinational, so mem_dout already shows the instruction word 0x95C9 during this state. ir_ld is high; on the clock edge leaving Fetch, ir_out steps from 0x0000 to 0x95C9.

Decode (state = 1): no control strobe is active. You can watch the decoded fields settle: the controller sees opcode 1001, and the register file read addresses take the values rs = 12, rt = 9 (meaningless for LW — they are just IR bit fields — which is exactly why nothing is write-enabled in this state).

Execute (state = 2): mem_addr_sel flips to 1 and mem_addr jumps from 0 to 201 (0xC9) — the address-mux handoff is one of the clearest things in the plot. mem_dout shows 25 (0x0019), rf_wdata_sel = 01 routes it to rf_wdata, and rf_we is high. On the exit edge, u_rf.rf[5] steps from 0 to 25.

UpdatePC (state = 3): pc_inc pulses and pc_out steps 0 → 1 on the exit edge, and the FSM wraps back to Fetch with mem_addr now following the new PC.

## 8.2 An ALU instruction: ADD r7, r5, r6 (instruction 2)

[ PLACE SCREENSHOT HERE ] Vivado waveform, ADD r7,r5,r6. Signals: clk, state, ir_out, rp_addr, rq_addr, rp_data, rq_data, alu_op, alu_y, rf_we, u_rf.rf[7].

Figure 8-2: Vivado waveform, ADD r7,r5,r6. Signals: clk, state, ir_out, rp_addr, rq_addr, rp_data, rq_data, alu_op, alu_y, rf_we, u_rf.rf[7].

By Fetch of instruction 2, r5 = 25 and r6 = 35 are already in the register file. After the IR captures 0x0756, the read addresses become rp_addr = 5 and rq_addr = 6, and rp_data/rq_data show 25 and 35 for the whole Decode state — the settle time that state exists to provide. In Execute, alu_op = 00 and alu_y shows 60 combinationally; rf_we is the only enable high, and r7 becomes 60 on the exit edge. Also visible here: RF_Rp_zero is low (25 ≠ 0) — the flag tracks port P continuously even though nothing consumes it for an ADD.

## 8.3 A store and the program overview: SW r7, 203 and the full run

[ PLACE SCREENSHOT HERE ] Vivado waveform, SW r7,203 Execute step. Signals: clk, state, ir_out, rf_rp_addr_sel, rp_addr, rp_data, mem_addr, mem_we, din, u_mem.ram[203].

Figure 8-3: Vivado waveform, SW r7,203 Execute step. Signals: clk, state, ir_out, rf_rp_addr_sel, rp_addr, rp_data, mem_addr, mem_we, din, u_mem.ram[203].

The store shows the two SW-specific muxes doing their jobs at once: in Execute, rf_rp_addr_sel = 1 swings rp_addr to IR[11:8] = 7 (instead of IR[7:4], which holds 12 for this word), so rp_data presents 60; and mem_addr_sel = 1 puts 203 (0xCB) on the address bus while mem_we pulses. ram[203] updates to 60 on the exit edge. The same pattern repeats for SW r4, 204 (225) and SW r2, 205 (255) later in the run.

[ PLACE SCREENSHOT HERE ] Vivado waveform, full-program overview. Signals: clk, state, pc_out, ir_out, done, u_mem.ram[203], u_mem.ram[204], u_mem.ram[205].

Figure 8-4: Vivado waveform, full-program overview. Signals: clk, state, pc_out, ir_out, done, u_mem.ram[203], u_mem.ram[204], u_mem.ram[205].

The overview plot is the money shot: pc_out staircases 0 through 10, the three RAM locations step to 0x3C, 0xE1, 0xFF at instructions 3, 6, and 9, and after the fetch of address 10 (ir_out = 0xF000) the state register enters Halt and done goes high and stays there. PC freezes at 10 — the UpdatePC step never runs for HALT, which is the intended behavior.

## 8.4 Testbench console output

The testbench prints one PASS/FAIL line per checked register and memory location. Expected output of a passing run:

=== EE 5193 RISC testbench ===

--- program halted at PC = 10 after 495000 ---

PASS  r5 = 25

PASS  r6 = 35

PASS  r7 = 60

PASS  r8 = 250

PASS  r4 = 225

PASS  r3 = 30

PASS  r2 = 255

PASS  mem[203] = 60 (0x3c)

PASS  mem[204] = 225 (0xe1)

PASS  mem[205] = 255 (0xff)

=== ALL CHECKS PASSED ===

[ PLACE SCREENSHOT HERE ] Vivado Tcl console after Run Behavioral Simulation, showing the PASS lines and “=== ALL CHECKS PASSED ===”.

Figure 8-5: Vivado Tcl console after Run Behavioral Simulation, showing the PASS lines and “=== ALL CHECKS PASSED ===”.

# 9. Board Demonstration Photographs

Bring-up procedure: generate the bitstream, program the board from the Hardware Manager, and press BTNC once to reset. The display shows dashes (-- -- --) for the few microseconds the program needs — perceptually instant — and then settles on 3C  E1  FF, which is mem[203] = 60, mem[204] = 225, and mem[205] = 255 in hexadecimal, with LED0 lit to indicate the HALT state was reached. Holding BTNC down keeps the machine in reset, so the dashes are visible for as long as the button is held; releasing it re-runs the program from scratch.

[ PLACE BOARD PHOTO HERE ] Nexys A7 running the design: 7-segment display showing “3C E1 FF”, LED0 lit.

Figure 9-1: Nexys A7 running the design: 7-segment display showing “3C  E1  FF”, LED0 lit.

[ PLACE BOARD PHOTO HERE ] Close-up of the 7-segment display: mem[203]=0x3C, mem[204]=0xE1, mem[205]=0xFF with blank separator digits.

Figure 9-2: Close-up of the 7-segment display: mem[203]=0x3C, mem[204]=0xE1, mem[205]=0xFF with blank separator digits.

[ PLACE BOARD PHOTO HERE ] BTNC held down: display shows dashes (machine in reset, done low) — demonstrates the done-gated display.

Figure 9-3: BTNC held down: display shows dashes (machine in reset, done low) — demonstrates the done-gated display.

# 10. Problems Encountered and How I Resolved Them

## 10.1 The SRA encoding in the handout is inconsistent (found in simulation)

My first full-program simulation failed exactly one check: FAIL mem[205] = 225, expected 255 — the display would have shown E1 twice. Working backwards: mem[205] comes from r2, r2 comes from XOR r2, r3, r4, and the waveform showed the XOR reading rp_data = 0 for r3. r3 was never written. But the SRA result (30) had landed in r2, where the XOR then overwrote it with 0 ^ 225 = 225. The rd field of the handout's SRA word 0110_0010_0111_0000 is 0010 — r2, not the r3 the mnemonic SRA 3 7 claims. Since the following XOR explicitly reads r3, the mnemonic has to be right and the binary wrong, so I corrected the word to 0110_0011_0111_0000 in program.mem (with a comment marking the change) and re-ran: all checks passed. Good reminder that a self-checking testbench catches what eyeballing a waveform misses — I had scrolled past that XOR several times without noticing r3 was never written.

## 10.2 Registered-read memory broke the four-step model (everything off by one)

My first memory used the registered-read template so Vivado would infer block RAM. In simulation the program did nothing sensible: the first instruction executed as 0x0000 (which my decoder treats as ADD r0, r0, r0) and every subsequent instruction was the previous one. The cause: with a synchronous read, the data for address PC appears one clock after Fetch applies the address, so ir_ld was capturing the read-data register's stale contents. LW had the same one-cycle problem in its Execute state. The clean fixes are either to add a wait state to Fetch and to LW (making them two-state steps, which breaks the assignment's four-step structure) or to use a combinational read. I chose the combinational read: Vivado infers distributed LUT RAM, which at 256 × 16 is a trivial fraction of an XC7A100T, and the four-state execute model stays exact. The lesson I'm taking away: the BRAM-vs-LUTRAM decision is not just a resource question, it changes the cycle-level architecture.

## 10.3 Blocking assignment in the register file write port

Early on I had written the register file write as a blocking assignment (rf[waddr] = wdata;). Behavioral simulation looked fine, but it is a real race: with blocking assignments, whether another always block sampling the register file on the same edge sees the old or new value depends on simulator evaluation order, not on the design. In this design the ALU operands are sampled through combinational reads by the register write that happens on the same Execute exit edge — with the blocking write, a same-edge reader could observe the new value a delta cycle early, which is not what the flip-flop hardware will do. It surfaced as a mismatch between behavioral and post-synthesis functional simulation on the back-to-back pair ADD r7, r5, r6 / SW r7, 203. Changing the write to a non-blocking assignment (rf[waddr] <= wdata;) made both simulations agree. Rule I now follow without exception: sequential logic gets non-blocking assignments, period.

## 10.4 Seven-segment ghosting and phantom zeros

First hardware bring-up of the display showed two artifacts: faint copies of each digit on its neighbor (“ghosting”), and the separator digits showing dim zeros instead of being dark. The ghosting came from my original scan rate — I had grabbed counter bits [16:14], and combined with the cathode pattern changing a fraction before the anode switched, each digit's segments briefly drove the next digit. Two fixes together solved it: I registered an and seg in the same always block so they change on the same edge, and slowed the scan to counter bits [19:17] (1.31 ms per digit, ~95 Hz frame rate — no visible flicker). The phantom zeros were a logic bug: I had blanked unused digits by feeding them nibble 0 instead of deasserting their anodes. The fix was the digit_en mask, which turns the anode off entirely for blanked positions. That mask then turned out to be exactly what I needed for the two dark separator columns in the 3C  E1  FF layout.

## 10.5 Divided clock vs. clock enable

To slow the CPU down I first did the obvious thing: a counter whose MSB toggled at ~1 MHz, used directly as the CPU clock. It worked in behavioral simulation, but Vivado flagged the divided net as a gated/derived clock (routed on fabric instead of a clock buffer), and the display logic — still on the raw 100 MHz clock — now crossed clock domains into the CPU's memory port with no synchronization, which showed up as critical warnings in the timing report. Rather than constrain a generated clock and add CDC synchronizers for a toy processor, I restructured to the standard FPGA idiom: everything clocks on the one 100 MHz clock, and the core's sequential elements are qualified by a one-cycle-in-a-hundred clock-enable pulse. All the cross-domain paths disappeared (there is only one domain), timing closed with large margin, and the CPU rate is still a one-line parameter change.

## 10.6 $readmemb file not found — in synthesis but not simulation

Behavioral simulation loaded program.mem fine, but my first synthesis run warned that the file could not be found and happily built a memory of all zeros. On the board that meant done never rose (an all-zero memory executes ADD r0, r0, r0 forever with the PC wrapping around), so the display sat on dashes indefinitely — my first “is the FPGA even alive?” moment of the project, resolved by reading the synthesis log instead of guessing. Simulation and synthesis resolve relative paths differently: xsim ran from a directory where my path happened to work, synthesis did not. The robust fix was to keep a bare filename in $readmemb and add program.mem to the Vivado project itself as a design source (it also has to be in the simulation fileset), so both tools resolve it through the project rather than the working directory. After that, both flows loaded the same image, and the post-synthesis functional simulation matched behavioral.

# 11. Conclusion

The processor meets the assignment as specified: every block is its own Verilog module, the controller takes only the IR and RF_Rp_zero and walks each instruction through Fetch, Decode, Execute, and UpdatePC, and the demo program runs from the unified memory with operands 25 and 35 at locations 201/202, leaving 60, 225, and 255 in locations 203–205 and on the 7-segment display as 3C E1 FF. The self-checking testbench verifies all seven result registers and all three memory results against the hand calculation, and the design synthesizes into a single 100 MHz clock domain on the Nexys A7 with no latches and no timing violations. Beyond the requirements, the exercise of giving RF_Rp_zero a real consumer (the JZ branch) and a defined stopping point (HALT) made the controller table and ASM-D charts feel like a complete, if small, ISA rather than a fixed-program sequencer — and the bugs in Section 10 taught me more about the simulation-to-hardware gap than the parts that worked on the first try.